RESEARCH SUMMARY | PHASE 7B

Reliable Robotic Manipulation Through Evidence-Grounded Control

Across identical simulation protocols, optimized PID produced the strongest aggregate reliability while no controller dominated every condition.

# Problem and research question

Reliable manipulation requires more than nominal trajectory tracking: controllers must remain interpretable under payload, configuration, disturbance, noise, and actuator changes.

# Method

A 2-link arm compares manual PID, Mamdani Fuzzy-PID, and bounded optimization-assisted PID under identical 0.001 s simulation, trajectory, torque-limit, threshold, and paired-seed protocols.

# Key results

Evidence snapshot: 7.283% nominal objective reduction; 10/13 optimized PID deterministic scenarios passed; 30/30 vs 0/30 per controller-scenario cell: isolated-noise cell vs full-combined-stress cell

| 7.283% nominal objective reduction | 10/13 optimized PID deterministic scenarios passed | 30/30 vs 0/30 per controller-scenario cell: isolated-noise cell vs full-combined-stress cell |
| --- | --- | --- |

Figure. Deterministic stress testing separates aggregate controller reliability while preserving failed cases.

# Why this matters

The project supplies a transparent low-level execution layer that can sit beneath future perception, language, and task-planning systems.

# Simulation scope

Simulation only; no hardware fidelity, collision avoidance, perception, online safety supervisor, or end-to-end embodied-AI implementation is claimed.

# Limitations and next steps

Validate on hardware, add calibrated sensing uncertainty, and introduce runtime safety monitoring before deployment claims.

Sources: docs/report/technical_report.md | results/report/report_evidence.json | docs/report/build_manifest.json